Sodium capacity vs Lead Acid vs LFP vs LTO

Usable capacity during an alternator-out emergency:

Sodium-ion cells do have a different voltage profile from LFP, but published  testing shows that approximately 80% of usable capacity is between 3.1 and 2.6 V per cell

Q: At the aircraft's actual continuous electrical load, how many amp-hours can the battery deliver before the aircraft's minimum acceptable bus voltage – 11 volts - is reached?

Lead-acid has a significant voltage decline under load, and it also suffers from the Peukert effect, meaning its available capacity decreases as discharge current increases. Lead-acid batteries are commonly rated at the relatively gentle 20-hour rate (0.05C) precisely because their usable capacity falls at higher discharge rates.

The relevant comparison isn't sodium versus LFP. It is sodium versus the AGM battery being replaced, at the actual aircraft emergency load and minimum acceptable bus voltage.

At 15 A, a 16-Ah PC680 does not deliver its full 16 Ah; Odyssey's own discharge data shows approximately 11.7 Ah at 15.6 A.  A 17-Ah sodium battery's discharge curve indicates approximately 12 Ah is available above an 11V  bus threshold at approximately a 0.9C discharge rate. Therefore, any assertion that sodium requires twice the capacity is not supported by the actual discharge data.

| Battery | Nominal Ah | Approx. usable Ah above 11 V | Approx. endurance @ 15 A |
| --- | --- | --- | --- |
| PC680 AGM | 16 Ah | ~11–12 Ah | ~45–48 min |
| ETX900 LFP | 15.4 Ah | ~15 Ah | ~60 min |
| Sodium | 17 Ah | ~12 Ah | ~48–50 min |

A sodium battery at 17 Ah could actually provide roughly the same or somewhat greater 11V emergency endurance as the 16-Ah PC680, despite the sodium voltage curve being considerably less flat than LFP.

And that's because the PC680 itself loses a substantial portion of its nameplate capacity at a ~1C discharge rate.

Odyssey's own published data makes that very clear: the PC680 delivers 16 Ah at 8–10 hour/20-hour rates, but only 11.7 Ah at its 45-minute/15.6-A point.

A 17-Ah sodium battery is in the same general endurance territory as the 16-Ah PC680—not remotely 2× larger.

And the ETX900 advantage is only about 25% more endurance, not 100% more

At the actual 15-A emergency load, the relevant comparison is usable endurance above the aircraft's minimum bus voltage—not nameplate Ah and not sodium-versus-LFP voltage flatness. The 17-Ah sodium battery provides approximately the same 11-V emergency endurance as the 16-Ah PC680 AGM, while the 15.4-Ah ETX900 LFP provides roughly 25% more. The data do not indicate a requirement for twice the sodium capacity.

### Compare the four batteries

| Battery | Nominal capacity | Approx. capacity available after 14.4-V operation | 15-A theoretical time |
| --- | --- | --- | --- |
| Odyssey PC680 | 16 Ah | Near 100% | 45-48 min nominal |
| EarthX ETX900 | 15.6 Ah | Near 100% | ~60 min nominal |
| 4S Sodium | 17 Ah | Near 80% | 48-50 min nominal |
| 6S Toshiba LTO | 20 Ah | ~15.4 Ah at 2.40 V/cell/80% | ~62 min |

### 1. AGM — the conventional choice

Odyssey PC680

Advantages:

- Proven technology

- Simple

- Familiar to mechanics and aircraft owners

- No sophisticated battery electronics required

- Well-understood failure behavior

Disadvantages:

- Heavy

- Significant voltage sag under high current

- Lead-acid capacity is strongly dependent on discharge rate

- Less usable capacity at high discharge rates

- Shorter useful service life compared with LTO

- More mass for the same starting/emergency-storage capability

The PC680 is a 16-Ah battery, but Odyssey's published discharge data show the practical capacity falling substantially at higher discharge rates. That's why simply saying "16 Ah" doesn't tell the whole emergency-power story.

# 2. LFP — the high-power lithium choice

ETX900

the ETX900-TSO is specified at:

- 13.2 V

- 15.6 Ah

- 800 A peak power

- 600 A rated power

- 31 A maximum continuous discharge

- 13.9–14.6 V charging

- 5.4 lb

- FAA TSO-C179b

Its big selling point is power density.

For aircraft starting:

LFP gives you tremendous starting power in a relatively small package.

But it achieves this with a sophisticated battery-management system. EarthX documents BMS functions including overcharge, over-discharge, excessive-cranking, high-temperature and short-circuit protection.

# 3.  Sodium — your alternative to LFP

Sodium provides an alternative high-power battery architecture without making the customer choose LFP simply because it is the only modern lithium-type option available.

### High starting power

A Sodium battery is engineered around the actual aircraft starting requirement, rather than maximizing Ah.

For a starter battery, that's important.

The aircraft doesn't care whether the battery has 10Ah or 16 Ah or 20 Ah when the starter needs hundreds of amps.

It cares about:

Voltage × current × time.

### Simple architecture

A Sodium battery has:

- No BMS

- No internal contactors

- No electronic switching

- No internal protection electronics

- A high-power sodium battery using a simple, passive battery architecture.

# 4. LTO — your long-life/high-endurance alternative

The Toshiba SCiB has characteristics that are unusually well suited to an aircraft battery application.

Toshiba specifically states that SCiB tolerates continuous constant-voltage float charging, which is extremely relevant to an aircraft electrical system. Toshiba also reports more than 70% capacity retention after 20,000 cycles under specified conditions.

### 14.6-V aircraft bus → 2.43 V/cell → ~80% SOC

That means the LTO doesn't need to reach a conventional "full charge" voltage to have substantial stored energy.

LTO sacrifices some energy storage at the aircraft's normal 14.6-V operating point, but still retains approximately 15.6 Ah from a 20-Ah cell, while offering exceptionally long cycle life and excellent high-current capability.

Toshiba rates the 20-Ah high-energy SCiB at 1,200 W output at 50% SOC for 10 seconds at 25°C.

15 A × ~13 V ≈ 195 W.

So an alternator out scenario with 15A load is nowhere near the LTO cell's high-power capability.

|  | AGM | LFP | NFPP Sodium | LTO |
| --- | --- | --- | --- | --- |
| Starting power | Good | Excellent | Excellent | Excellent |
| Emergency endurance | Good | Very good | Good | Excellent |
| Weight | Poor | Excellent | Excellent | Good |
| Cycle life | Fair | Good | Good | Exceptional |
| Float/continuous charging | Excellent | Good | Very good | Excellent |
| Electronics required | None | BMS | None | None required at cell level |
| Thermal/failure architecture | Conventional | Requires BMS/thermal controls | Different chemistry | Very stable LTO chemistry |
| Best reason to buy | Familiarity | Maximum power/weight | Simple high-power alternative | Long life + endurance + charging tolerance |